# TABLE OF CONTENTS

Confidentiality Statement Declaration

Acknowledgement Abstract

Executive Summary Table of Contents Table of Figures

List of Abbreviations

## CHAPTER 1: INTRODUCTION AND MOTIVATION

  - Background

  - Aerospace Certification Context (DRDO – CEMILAC Perspective)

  - Need for Digital Defect Intelligence

  - Role of Artificial Intelligence in Defect Analysis

  - Motivation for the Present Work

  - Scope of the Project

  - Expected Impact

## CHAPTER 2: PROBLEM STATEMENT AND OBJECTIVES

  - Problem Statement

  - Objectives of the Project

  - Deliverables

  - Constraints and Assumptions

  - Acceptance Criteria

## CHAPTER 3: SYSTEM ARCHITECTURE AND METHODOLOGY

  - Overview of the Proposed Framework

  - Data Processing Pipeline

  - Data Ingestion and Extraction

  - Data Transformation and Structuring

  - Data Aggregation and Storage

  - Feature Engineering and Dataset Preparation

  - AI-Based Analytics and Processing

  - Risk Scoring Methodology

  - Similarity Search Mechanism

  - Root Cause Clustering Approach

  - System Workflow Diagram

## CHAPTER 4: IMPLEMENTATION DETAILS

  - Project Structure and Modules

  - Data Ingestion Module

  - Field Extraction Module

  - Data Merging and Dataset Creation

  - Analytics and Model Implementation

  - Similarity Search Implementation

  - Risk Scoring Implementation

  - Dashboard Development

  - Offline Deployment Strategy

## CHAPTER 5: RESULTS AND ANALYSIS

  - Dataset Overview

  - Root Cause Clustering Analysis

  - Risk Band Distribution Analysis

  - Risk Score Distribution

  - System-wise Defect Analysis

  - Trend Analysis

  - Performance Evaluation of Models

## CHAPTER 6: DISCUSSION

  - Key Findings

  - Insights from Data Analysis

  - Advantages of the Proposed System

  - Limitations of the System

## CHAPTER 7: CONCLUSION AND FUTURE WORK

  - Conclusion

  - Future Enhancements

Appendices

Code Structure Overview

CHAPTER 1

INTRODUCTION

Aircraft safety and operational reliability are critical aspects of modern aviation systems. Fighter aircraft in particular operate in extremely demanding environments that require high engine performance, precise control systems, and robust monitoring mechanisms. During flight operations, aircraft engines experience intense mechanical loads, high rotational speeds, and extreme thermal conditions. Maintaining engine health under these circumstances is essential to ensure mission success and pilot safety.

With the advancement of aviation technology, modern aircraft are equipped with numerous sensors that continuously monitor engine performance parameters such as temperature, fuel flow rate, oil pressure, vibration levels, and rotational speed. These sensors generate large volumes of telemetry data during flight operations. Analyzing this telemetry data can provide valuable insights into engine performance and help identify early signs of mechanical failure.

Traditionally, aircraft monitoring systems relied on periodic inspections and threshold-based alert systems. These systems generate warnings when a particular parameter exceeds a predefined safety limit. However, such methods often fail to capture complex relationships between multiple parameters that may collectively indicate abnormal engine behavior. As a result, potential faults may remain undetected until they become critical.

The rapid growth of data analytics, artificial intelligence, and machine learning technologies has enabled new approaches for analyzing large datasets and detecting hidden patterns in complex systems. These technologies allow engineers to analyze aircraft telemetry data more effectively and identify anomalies that could indicate early stages of engine degradation.

In this project, a Fighter Aircraft Flight Readiness System (FAFRS) is developed to analyze aircraft telemetry data using data analytics techniques. The system processes telemetry datasets, calculates engine health indicators, detects anomalies using machine learning algorithms, and generates a mission readiness decision.

The system evaluates aircraft readiness and classifies it into three categories:

- Ready for Flight

- Mission with Caution

- Not Ready – Maintenance Required

By automating the analysis of telemetry data, the FAFRS system can assist maintenance teams and operational planners in making informed decisions about aircraft readiness. This approach has the potential to enhance aircraft safety, improve maintenance efficiency, and support predictive maintenance strategies in aviation systems.

1.1 Background of the Study

Modern fighter aircraft operate under highly demanding environmental and mechanical conditions that require continuous monitoring of engine performance and flight behaviour. Aircraft propulsion systems are exposed to high temperature gradients, rapid rotational motion, aerodynamic loads, and pressure variations throughout different flight phases such as taxiing, takeoff, climb, cruise, descent, and landing. Monitoring telemetry parameters during these phases is essential for ensuring safe operation and maintaining mission readiness.

Traditionally, aircraft monitoring systems relied on threshold-based warning mechanisms that generated alerts whenever individual sensor values exceeded predefined limits. Although effective for detecting critical failures, these systems were not capable of identifying complex interactions between multiple parameters that collectively indicate early-stage degradation. With the increasing availability of high-frequency telemetry data generated by onboard sensors, advanced analytical techniques have become necessary to interpret system behaviour more accurately.

Recent developments in data analytics and machine learning technologies have enabled intelligent monitoring frameworks capable of identifying hidden patterns in telemetry datasets. These approaches allow engineers to evaluate aircraft engine condition automatically and support predictive maintenance strategies that reduce operational risk.

1.2 Problem Statement

Aircraft engines operate under severe mechanical and thermal stress during flight operations, which increases the probability of performance degradation over time. Traditional monitoring approaches analyze parameters independently and often fail to identify combined effects of multiple deviations occurring simultaneously. As a result, early warning signs of system deterioration may remain undetected until they develop into critical faults.

There is therefore a need for an intelligent monitoring system capable of analyzing telemetry datasets collectively and identifying abnormal behaviour patterns automatically. Such a system can support maintenance personnel in making informed readiness decisions and improve aircraft availability by enabling predictive maintenance strategies.

1.3 Objectives of the Project

The primary objective of this project is to design and implement a telemetry-based Fighter Aircraft Flight Readiness System capable of analyzing engine and flight performance parameters using machine learning techniques. The system detects abnormal parameter behaviour, computes readiness scores, and classifies aircraft operational condition into readiness categories that support maintenance planning and mission execution decisions.

1.4 Scope of the Project

The scope of this project includes analysis of telemetry datasets containing engine and flight performance parameters such as airspeed, altitude, pitch angle, engine RPM, exhaust gas temperature, vibration levels, oil pressure, and fuel flow rate. The system evaluates variations in these parameters across flight phases and generates readiness classifications based on anomaly detection results. The project focuses on predictive monitoring of propulsion system behaviour and does not include structural or avionics subsystem diagnostics.

CHAPTER 2

LITERATURE REVIEW

Aircraft health monitoring has become an essential area of research in modern aviation systems. With the increasing complexity of aircraft engines and avionics systems, traditional monitoring techniques have become insufficient for detecting complex failure patterns. Aircraft engines operate under highly demanding conditions involving extreme temperatures, high rotational speeds, and rapid pressure changes. These operational conditions increase the likelihood of component wear and mechanical failures.

In order to ensure safe and reliable aircraft operations, continuous monitoring of engine performance parameters is required. Aircraft are equipped with multiple sensors that collect telemetry data related to engine performance. This telemetry data provides valuable information about the operational state of the engine and can be used to detect abnormalities and potential failures.

This chapter reviews existing research and technologies related to aircraft health monitoring, predictive maintenance, and machine learning–based anomaly detection techniques.

2.1 Aircraft Health Monitoring Systems

Aircraft health monitoring systems have become essential components of modern aviation platforms due to the increasing complexity of propulsion systems and onboard instrumentation. These systems continuously collect telemetry measurements from sensors installed across engine and flight subsystems and analyze variations in parameter behaviour to detect abnormal operating conditions. Continuous monitoring enables early identification of performance degradation and supports preventive maintenance activities before faults become critical.

Conventional monitoring systems relied heavily on rule-based diagnostic frameworks that generated alerts whenever sensor readings exceeded predefined thresholds. While effective for detecting major deviations, these systems lacked the capability to analyze relationships between multiple parameters simultaneously. As aircraft systems became more sophisticated, the need for intelligent monitoring approaches capable of analyzing large telemetry datasets increased significantly.

2.2 Predictive Maintenance in Aviation

Predictive maintenance strategies were introduced to overcome limitations of traditional maintenance approaches that depended on fixed inspection schedules. Instead of performing maintenance at predetermined intervals, predictive maintenance systems evaluate equipment condition continuously and recommend servicing only when required. This improves aircraft availability, reduces maintenance cost, and minimizes unexpected failures during mission operations.

Telemetry data collected from onboard sensors plays an important role in predictive maintenance systems because it provides real-time insight into engine performance behaviour. By analyzing historical and real-time datasets, predictive algorithms can identify abnormal performance trends and estimate remaining useful life of critical components.

2.3 Telemetry Data Analysis

Telemetry refers to the automatic measurement and transmission of data from remote systems to monitoring stations for analysis. In aviation systems, telemetry plays a critical role in monitoring aircraft performance during flight operations.

Aircraft engines are equipped with multiple sensors that continuously collect data related to engine performance and environmental conditions. This data is transmitted to onboard monitoring systems or ground stations through telemetry communication channels.

The analysis of telemetry data allows engineers to monitor engine performance, detect abnormal patterns, and evaluate system health.

The process of telemetry data analysis typically involves several steps:

- Data Collection: Sensor data is collected from aircraft engines and transmitted through telemetry systems.

- Data Preprocessing: Raw telemetry data is cleaned to remove errors, missing values, and noise.

- Feature Extraction: Meaningful features are derived from raw sensor readings to represent system behavior.

- Pattern Recognition: Analytical techniques are used to identify patterns in the telemetry data.

- Anomaly Detection: Machine learning algorithms are applied to detect abnormal behavior in the system.

Programming languages such as Python are widely used for telemetry data analysis due to their powerful data processing libraries. Tools such as Pandas, NumPy, and Scikit-learn provide efficient frameworks for data manipulation, statistical analysis, and machine learning.

2.4 Machine Learning in Aircraft Monitoring

Machine learning techniques have been widely applied in aircraft telemetry analysis to identify abnormal patterns and classify system behavior automatically. Algorithms such as Support Vector Machines, Decision Trees, Neural Networks, and clustering techniques have demonstrated strong performance in classification and anomaly detection tasks involving aviation datasets.

Isolation Forest is particularly suitable for telemetry analysis because it isolates abnormal observations efficiently without requiring labelled training data. Since aircraft telemetry datasets rarely include explicit fault labels, unsupervised learning algorithms provide practical solutions for monitoring propulsion system behavior.

Several machine learning techniques have been applied in aircraft telemetry analysis, including:

- Isolation Forest: Used for anomaly detection in large datasets.

- Support Vector Machines (SVM): Used for classification and anomaly detection tasks.

- Neural Networks: Used for complex pattern recognition and predictive modeling.

- Clustering Algorithms: Used to group similar data points and identify unusual patterns.

- Decision Trees: Used for classification and decision-making tasks.

2.5 Visualization-Based Monitoring Systems

Visualization dashboards improve interpretability of telemetry datasets by presenting parameter variations across flight phases in graphical form. These dashboards allow engineers to observe performance trends quickly and identify deviations that may indicate abnormal system behavior. Integration of visualization modules with anomaly detection algorithms enhances monitoring accuracy and supports effective readiness evaluation.

CHAPTER 3

SYSTEM ARCHITECTURE AND METHODOLOGY

The development of an intelligent aircraft monitoring system requires a structured architecture capable of processing large volumes of telemetry data efficiently. The Fighter Aircraft Flight Readiness System (FAFRS) is designed to analyze aircraft engine telemetry data using data analytics and machine learning techniques to determine whether an aircraft is ready for its next mission.

The system architecture integrates several functional modules that work together to collect, process, analyze, and interpret telemetry data generated by aircraft sensors. These modules ensure that raw sensor data is transformed into meaningful insights that help maintenance engineers and operational teams evaluate aircraft readiness.

The methodology followed in this project includes data ingestion, data validation, feature engineering, anomaly detection using machine learning algorithms, health index computation, visualization of system behavior, and final decision generation. Each stage in the pipeline plays a critical role in ensuring that the system produces reliable and accurate readiness assessments.

This chapter describes the architecture of the FAFRS system, the workflow of the data analysis process, and the methods used for telemetry data processing and anomaly detection.

3.1 Overview of the Proposed System

The Fighter Aircraft Flight Readiness System is designed as a telemetry-driven monitoring platform capable of analyzing aircraft sensor datasets and generating readiness decisions automatically. The system follows a structured processing pipeline in which telemetry data flows sequentially through ingestion, preprocessing, feature transformation, anomaly detection, visualization, and readiness classification stages.

Telemetry datasets are uploaded through a Streamlit-based interface that enables users to interact with the system easily. Each dataset contains timestamped observations representing variations in engine performance and flight attitude parameters across different mission phases. After ingestion, validation procedures ensure that required telemetry parameters are present and formatted correctly before further analysis begins.

Fig 3.1 – System Architecture Flow

This structured pipeline ensures that aircraft telemetry data is processed systematically, allowing the system to generate reliable mission readiness assessments.

3.2 Data Processing Workflow

The preprocessing stage improves dataset quality by handling missing values, removing noise, and normalizing parameters measured in different units. Normalization ensures uniform contribution of each parameter to anomaly detection results and improves model performance. Feature engineering techniques further enhance dataset representation by identifying derived indicators that capture performance deviations more effectively.

The Isolation Forest algorithm is then applied to identify abnormal telemetry observations within the dataset. The algorithm isolates anomalies by constructing decision trees that separate abnormal data points from normal observations. Anomaly scores generated during this process are aggregated across parameters to compute an overall readiness score representing aircraft engine condition.

Fig 3.2 – System Architecture Flow

📌 Insert Screenshot
Figure 3.1 Fighter Aircraft Flight Readiness System Dashboard Showing Readiness Score

3.3 Readiness Score Generation

The readiness score represents the combined effect of deviations detected across telemetry parameters during flight operations. Higher anomaly severity results in lower readiness scores, while stable parameter behavior produces higher readiness classifications. Based on the computed score, the aircraft is categorized into readiness levels that support maintenance decision-making and mission planning activities.

CHAPTER 4

DATASET DESCRIPTION AND IMPLEMENTATION

4.1 Telemetry Dataset Description

The dataset used in this project consists of time-series telemetry measurements recorded across multiple flight phases including taxi-out, takeoff, climb, cruise, descent, landing, and taxi-in. Each observation includes values representing engine performance indicators and flight attitude parameters that collectively describe aircraft operational behaviour during mission execution.

These parameters provide insight into propulsion system stability and allow identification of deviations that may indicate abnormal operating conditions. Phase-wise segmentation of telemetry data enables analysis of parameter behaviour under varying aerodynamic and mechanical loading conditions.

Each of these parameters plays an important role in monitoring engine performance and detecting abnormal operating conditions.

For example:

- Engine RPM indicates the speed of the turbine rotation and helps detect abnormal engine load.

- Exhaust Gas Temperature (EGT) is used to monitor combustion efficiency and detect overheating.

- Oil pressure ensures proper lubrication of engine components.

- Vibration levels help detect mechanical imbalance or structural issues.

Monitoring these parameters simultaneously allows the system to evaluate the overall health of the engine.

4.2 Data Preprocessing

Before analysis begins, the dataset undergoes preprocessing to improve consistency and reliability. Missing values are handled using interpolation techniques that preserve continuity of telemetry trends. Extreme outliers caused by sensor noise are removed using statistical filtering methods. Normalization ensures that parameters measured in different physical units contribute equally to anomaly detection results.

4.3 System Implementation

The system was implemented using Python programming language supported by scientific computing libraries such as Pandas, NumPy, Scikit-learn, Plotly, and Matplotlib. Visualization dashboards were developed using Streamlit to provide an interactive interface for uploading telemetry datasets and observing readiness decisions in real time.

📌 Insert Screenshot
Figure 4.1 Parameter-wise Event Severity Count Graph

📌 Insert Screenshot
Figure 4.2 Distribution of Engine RPM and Exhaust Gas Temperature

CHAPTER 5

RESULTS AND ANALYSIS

5.1 Flight Phase Analysis

Phase-wise airspeed variation analysis revealed expected aerodynamic performance characteristics corresponding to different stages of aircraft operation. Lower airspeed values were observed during taxi phases, while higher values appeared during climb and cruise intervals. These observations confirmed dataset consistency and proper functioning of telemetry preprocessing procedures.

📌 Insert Screenshot
Figure 5.1 Airspeed Variation Across Flight Phases

5.2 Flight Attitude Analysis

Pitch angle behaviour analysis indicated stable aircraft attitude control during mission execution. Minor deviations observed during takeoff and descent phases remained within acceptable operational limits and therefore did not influence readiness classification results significantly.

📌 Insert Screenshot
Figure 5.2 Pitch Distribution Across Flight Phases

5.3 Engine Parameter Analysis

Fuel flow behaviour analysis demonstrated gradual variation corresponding to thrust demand transitions across flight phases. Engine RPM and exhaust gas temperature trends followed expected propulsion system behaviour patterns and confirmed stability of combustion processes during flight operations.

📌 Insert Screenshot
Figure 5.3 Fuel Flow Dynamic Analysis Output

📌 Insert Screenshot
Figure 5.4 Engine RPM and Exhaust Gas Temperature Trends

5.4 Correlation and Anomaly Analysis

Correlation heatmap visualization revealed strong relationships between propulsion parameters including engine RPM, exhaust gas temperature, and fuel flow rate. Anomaly distribution analysis indicated that most deviations occurred during climb and cruise intervals due to sustained engine loading conditions, although these deviations remained within acceptable limits.

📌 Insert Screenshot
Figure 5.5 Correlation Heatmap of Telemetry Parameters

📌 Insert Screenshot
Figure 5.6 Anomaly Type Distribution Across Flight Phases

CHAPTER 6

DISCUSSION

6.1 Interpretation of Results

The results obtained from telemetry analysis demonstrate that the Fighter Aircraft Flight Readiness System effectively identifies variations in propulsion system behaviour and evaluates aircraft readiness using machine learning–based anomaly detection techniques. By analyzing multiple telemetry parameters simultaneously, the system provides a more comprehensive understanding of engine condition compared to traditional threshold-based monitoring approaches that consider parameters independently. The readiness score generated from anomaly detection results reflects the overall operational stability of the aircraft and supports informed decision-making during mission preparation. The visualization modules integrated into the system further enhance interpretability of sensor data by presenting parameter variations clearly across different flight phases such as taxi, takeoff, climb, cruise, descent, and landing, allowing engineers to observe performance trends and identify deviations that may indicate abnormal behaviour. In addition, correlation analysis between engine parameters such as fuel flow rate, exhaust gas temperature, and engine RPM confirms expected coupling behaviour within propulsion subsystems, thereby strengthening confidence in the analytical framework used in the project and demonstrating the effectiveness of telemetry-driven readiness evaluation for predictive maintenance applications.

6.2 System Advantages

The proposed system provides several operational advantages in aircraft monitoring and readiness evaluation environments by enabling automatic identification of abnormal parameter behaviour without requiring manually labelled datasets, making it suitable for real-world aviation applications where labelled failure data is often limited or unavailable. Phase-aware trend analysis implemented in the system allows engineers to observe parameter behaviour during maneuver transitions and identify deviations associated with specific flight conditions, thereby improving diagnostic accuracy and supporting early detection of performance degradation before it develops into critical faults. The readiness scoring mechanism further simplifies interpretation by converting complex telemetry variations into a single quantitative indicator representing engine condition, while the integration of interactive visualization dashboards enables users to analyze telemetry data efficiently through intuitive graphical representations of parameter behaviour and anomaly distributions. As a result, the system improves situational awareness for maintenance personnel and operational planners and contributes to the implementation of proactive maintenance strategies in aviation monitoring environments.

6.3 Limitations of the Study

Although the system demonstrated reliable performance while analyzing simulated telemetry datasets, certain limitations remain in the current implementation because the evaluation was conducted using structured datasets representing predefined flight conditions rather than real-time sensor streams obtained from operational aircraft, and therefore the system’s performance under live flight conditions has not yet been fully validated. Another limitation involves the restricted number of telemetry parameters considered in the analysis, as inclusion of additional variables such as vibration spectrum data, combustion efficiency indicators, and structural load measurements could further improve readiness assessment accuracy and provide deeper insight into propulsion system health. Future integration with real aircraft telemetry acquisition systems and expansion of dataset diversity would therefore enhance the reliability, scalability, and operational applicability of the monitoring framework in practical aviation environments.

CHAPTER 7

CONCLUSION AND FUTURE SCOPE

7.1 Conclusion

The Fighter Aircraft Flight Readiness System developed in this project demonstrates the effectiveness of telemetry-driven anomaly detection techniques in evaluating aircraft engine condition and generating mission readiness decisions automatically by integrating machine learning algorithms with telemetry preprocessing and visualization modules to interpret complex sensor datasets across multiple flight phases. The readiness scoring mechanism provides a quantitative representation of aircraft operational condition that supports maintenance planning and mission scheduling decisions, while the application of the Isolation Forest algorithm enables efficient detection of abnormal telemetry observations without requiring labelled training data, making the approach suitable for aviation monitoring scenarios where failure-labelled datasets are limited. The visualization dashboards further improve interpretability of parameter variations and assist engineers in understanding performance trends during mission evaluation, thereby contributing to the development of intelligent aircraft monitoring frameworks that support predictive maintenance strategies and enhance operational safety in modern aerospace environments.

7.3 Future Work

Several improvements can be made to extend the capabilities of the FAFRS system in future research.

Integration with Real-Time Telemetry Systems

Future versions of the system could integrate with real-time telemetry streams from aircraft sensors. This would allow continuous monitoring of engine performance during flight operations and enable early detection of potential issues.

Advanced Machine Learning Models

More advanced machine learning algorithms such as deep learning models or ensemble learning techniques could be used to improve anomaly detection accuracy. These models may capture more complex relationships between telemetry parameters.

Predictive Maintenance

The current system focuses primarily on anomaly detection and readiness assessment. Future systems could incorporate predictive maintenance models capable of estimating the remaining useful life of engine components.

Integration with Aircraft Maintenance Systems

The system could be integrated with aircraft maintenance management systems to automatically schedule inspections and maintenance activities based on telemetry analysis results.

Enhanced Visualization and Dashboard Features

Future versions of the system could include more advanced dashboards with interactive analytics, historical performance comparisons, and predictive trend analysis.

# APPENDICES

## APPENDIX A: TELEMETRY DATASET DESCRIPTION

## The telemetry dataset used in this project consists of time-series sensor measurements representing aircraft engine performance and flight behaviour across different operational phases such as taxi-out, takeoff, climb, cruise, descent, landing, and taxi-in. The dataset includes parameters such as airspeed, altitude, pitch angle, engine RPM, exhaust gas temperature, fuel flow rate, oil pressure, vibration level, and ambient temperature. These parameters were preprocessed using normalization and noise removal techniques before applying anomaly detection.

## APPENDIX B: SAMPLE TELEMETRY DATASET STRUCTURE

This appendix presents a sample of the structured telemetry dataset used in the Fighter Aircraft Flight Readiness System. The dataset is organized in a tabular format, where each row represents a timestamped observation recorded during aircraft operation and each column corresponds to a specific engine or flight parameter.

Typical fields include:

• Timestamp
• Flight Phase
• Airspeed (kts)
• Altitude (ft)
• Pitch Angle (deg)
• Roll Angle (deg)
• Yaw Angle (deg)
• Engine RPM (%)
• Exhaust Gas Temperature (°C)
• Fuel Flow (kg/h)
• Oil Pressure (psi)
• Oil Temperature (°C)
• Vibration Level (g)
• Ambient Temperature (°C)
• Anomaly Type

## This structured dataset enables preprocessing, anomaly detection, correlation analysis, and readiness score computation for evaluating aircraft engine health and mission readiness.

## APPENDIX C: PROJECT FILE STRUCTURE

This appendix provides an overview of the project directory and file organization used during implementation of the Fighter Aircraft Flight Readiness System.

Example structure:

project/
│
├── data/
│ ├── raw_dataset/
│ ├── processed_dataset/
│
├── preprocessing.py
├── feature_engineering.py
├── anomaly_detection_model.py
├── readiness_score_calculation.py
├── visualization_dashboard.py
├── app.py
├── requirements.txt
└── README.md

This structured organization supports efficient dataset handling, anomaly detection processing, visualization generation, and readiness classification through the Streamlit-based dashboard interface.

## APPENDIX D: SAMPLE OUTPUTS AND VISUALIZATIONS

This appendix includes sample outputs generated by the Fighter Aircraft Flight Readiness System during telemetry data analysis and readiness evaluation. These outputs demonstrate the analytical capability of the system in identifying abnormal parameter behaviour and supporting mission readiness decisions.

Sample outputs include:

• Engine RPM trend analysis graphs
• Exhaust Gas Temperature distribution plots
• Fuel flow variation charts
• Airspeed variation across flight phases
• Pitch angle distribution graphs
• Correlation heatmaps of telemetry parameters
• Anomaly distribution by flight phase
• Parameter-wise event severity count graphs
• Mission readiness dashboard screenshots

## These outputs illustrate how telemetry parameters are analyzed and visualized to evaluate aircraft engine condition and generate readiness classification results.

## APPENDIX E: TOOLS AND TECHNOLOGIES USED

The following tools and technologies were used in the implementation of the Fighter Aircraft Flight Readiness System:

• Programming Language: Python

• Libraries: Pandas, NumPy, Scikit-learn, Matplotlib, Plotly

• Machine Learning Technique: Isolation Forest Algorithm

• Data Format: CSV Telemetry Dataset

• Visualization Platform: Streamlit Interactive Dashboard

These tools enabled efficient telemetry data preprocessing, anomaly detection, readiness score computation, and visualization of aircraft engine performance parameters through an interactive analytical interface.

# CODE STRUCTURE OVERVIEW

Ingest_simple.py

import re

from dataclasses import dataclass

from typing import Dict, List, Optional, Tuple

import numpy as np

import pandas as pd

import plotly.express as px

import plotly.graph_objects as go

import streamlit as st

st.set_page_config(page_title="Fighter Aircraft Flight Readiness System", layout="wide")

# ============================================================

# Fighter Aircraft Flight Readiness System (Category II)

# Stable demo version for the uploaded telemetry dataset

# ============================================================

# Expected dataset columns:

# timestamp, phase, airspeed_kts, altitude_ft, pitch_deg, roll_deg, yaw_deg,

# engine_rpm_pct, egt_c, fuel_flow_kgph, oil_pressure_psi, oil_temp_c,

# vibration_g, ambient_temp_c, anomaly_type

# ---------------------------

# Parameter rules

# ---------------------------

@dataclass

class ParameterRule:

key: str

display_name: str

group: str

unit: str

warn_low: Optional[float] = None

warn_high: Optional[float] = None

crit_low: Optional[float] = None

crit_high: Optional[float] = None

description: str = ""

RULES: Dict[str, ParameterRule] = {

"airspeed_kts": ParameterRule("airspeed_kts", "Airspeed", "Flight Profile", "kts", warn_low=20, warn_high=650, crit_low=0, crit_high=750, description="Aircraft speed envelope"),

"altitude_ft": ParameterRule("altitude_ft", "Altitude", "Flight Profile", "ft", warn_low=100, crit_low=50, description="Altitude trend"),

"pitch_deg": ParameterRule("pitch_deg", "Pitch", "Flight Attitude", "deg", warn_low=-25, warn_high=25, crit_low=-35, crit_high=35, description="Nose up/down attitude"),

"roll_deg": ParameterRule("roll_deg", "Roll", "Flight Attitude", "deg", warn_low=-60, warn_high=60, crit_low=-80, crit_high=80, description="Bank angle"),

"yaw_deg": ParameterRule("yaw_deg", "Yaw", "Flight Attitude", "deg", warn_low=-25, warn_high=25, crit_low=-35, crit_high=35, description="Yaw angle"),

"engine_rpm_pct": ParameterRule("engine_rpm_pct", "Engine RPM", "Engine Health", "%", warn_high=100, crit_high=106, description="Engine spool speed"),

"egt_c": ParameterRule("egt_c", "Exhaust Gas Temperature", "Engine Health", "°C", warn_high=780, crit_high=823, description="Exhaust thermal loading"),

"fuel_flow_kgph": ParameterRule("fuel_flow_kgph", "Fuel Flow", "Engine Health", "kg/h", warn_high=6500, crit_high=7500, description="Fuel consumption rate"),

"oil_pressure_psi": ParameterRule("oil_pressure_psi", "Oil Pressure", "Engine Health", "psi", warn_low=30, crit_low=25.7, description="Engine lubrication pressure"),

"oil_temp_c": ParameterRule("oil_temp_c", "Oil Temperature", "Engine Health", "°C", warn_high=130, crit_high=147, description="Oil system temperature"),

"vibration_g": ParameterRule("vibration_g", "Engine Vibration", "Engine Health", "g", warn_high=1.8, crit_high=2.4, description="Engine / airframe vibration level"),

"ambient_temp_c": ParameterRule("ambient_temp_c", "Ambient Temperature", "Environment", "°C", warn_low=-30, warn_high=45, crit_low=-40, crit_high=55, description="Outside air conditions"),

}

SEARCH_MAP = {

"airspeed": "airspeed_kts",

"speed": "airspeed_kts",

"altitude": "altitude_ft",

"height": "altitude_ft",

"pitch": "pitch_deg",

"roll": "roll_deg",

"yaw": "yaw_deg",

"rpm": "engine_rpm_pct",

"engine": "engine_rpm_pct",

"egt": "egt_c",

"temperature": "egt_c",

"engine temperature": "egt_c",

"fuel": "fuel_flow_kgph",

"fuel flow": "fuel_flow_kgph",

"oil pressure": "oil_pressure_psi",

"oil": "oil_pressure_psi",

"oil temp": "oil_temp_c",

"oil temperature": "oil_temp_c",

"vibration": "vibration_g",

"ambient": "ambient_temp_c",

"ambient temperature": "ambient_temp_c",

}

# ---------------------------

# Utilities

# ---------------------------

def normalize_text(text: str) -> str:

return re.sub(r"[^a-z0-9]+", " ", str(text).lower()).strip()

def preprocess(df: pd.DataFrame) -> pd.DataFrame:

df = df.copy()

df["timestamp"] = pd.to_datetime(df["timestamp"], errors="coerce")

df = df.sort_values("timestamp").reset_index(drop=True)

df["time_index"] = np.arange(len(df))

for col in RULES:

if col in df.columns:

df[col] = pd.to_numeric(df[col], errors="coerce")

if "phase" in df.columns:

df["phase"] = df["phase"].astype(str)

else:

df["phase"] = "unknown"

if "anomaly_type" in df.columns:

df["anomaly_type"] = df["anomaly_type"].astype(str)

else:

df["anomaly_type"] = "none"

return df

def classify_series(series: pd.Series, rule: ParameterRule) -> pd.Series:

sev = pd.Series("Normal", index=series.index)

if rule.warn_low is not None:

sev.loc[series < rule.warn_low] = "Warning"

if rule.warn_high is not None:

sev.loc[series > rule.warn_high] = "Warning"

if rule.crit_low is not None:

sev.loc[series < rule.crit_low] = "Critical"

if rule.crit_high is not None:

sev.loc[series > rule.crit_high] = "Critical"

return sev

def search_parameter(user_text: str, available_cols: List[str]) -> Optional[str]:

user_text = normalize_text(user_text)

if not user_text:

return None

for key, mapped in SEARCH_MAP.items():

if key in user_text and mapped in available_cols:

return mapped

for col in available_cols:

if normalize_text(col) in user_text or user_text in normalize_text(col):

return col

for col in available_cols:

rule = RULES.get(col)

if rule and user_text in normalize_text(rule.display_name + " " + rule.group + " " + rule.description):

return col

return None

# ---------------------------

# Event grouping and readiness

# ---------------------------

def group_consecutive_events(df: pd.DataFrame, col: str) -> pd.DataFrame:

if col not in df.columns:

return pd.DataFrame()

rule = RULES[col]

sev = classify_series(df[col], rule)

work = df[["timestamp", "phase", col]].copy()

work["severity"] = sev

work = work[work["severity"].isin(["Warning", "Critical"])].copy()

if work.empty:

return pd.DataFrame()

work["event_break"] = (

(work["severity"] != work["severity"].shift(1)) |

((work.index.to_series() - work.index.to_series().shift(1)) > 1)

)

work["event_id"] = work["event_break"].cumsum()

grouped = (

work.groupby("event_id", as_index=False)

.agg(

parameter=(col, lambda _: RULES[col].display_name),

severity=("severity", "first"),

start_time=("timestamp", "first"),

end_time=("timestamp", "last"),

phase=("phase", "first"),

peak_value=(col, "max"),

min_value=(col, "min"),

sample_count=(col, "size"),

)

)

grouped["duration_sec"] = (

grouped["end_time"] - grouped["start_time"]

).dt.total_seconds().fillna(0)

grouped["unit"] = RULES[col].unit

return grouped

def get_grouped_event_totals(df: pd.DataFrame) -> Tuple[int, int]:

critical_events = 0

warning_events = 0

for col in RULES:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

critical_events += int((grouped["severity"] == "Critical").sum())

warning_events += int((grouped["severity"] == "Warning").sum())

return critical_events, warning_events

def readiness_score(df: pd.DataFrame) -> Tuple[float, List[str]]:

penalties = 0.0

notes = []

important = [

"engine_rpm_pct", "egt_c", "fuel_flow_kgph", "oil_pressure_psi",

"oil_temp_c", "vibration_g", "pitch_deg", "roll_deg", "airspeed_kts"

]

for col in important:

if col not in df.columns:

continue

sev = classify_series(df[col], RULES[col])

warn_rate = (sev == "Warning").mean()

crit_rate = (sev == "Critical").mean()

penalties += warn_rate * 10 + crit_rate * 25

if crit_rate > 0.01:

notes.append(f"{RULES[col].display_name} has repeated critical excursions.")

elif warn_rate > 0.03:

notes.append(f"{RULES[col].display_name} shows warning-level deviations.")

score = max(0, round(100 - penalties, 1))

return score, notes[:6]

def get_readiness_status(df: pd.DataFrame) -> Tuple[float, str]:

score, _ = readiness_score(df)

crit_total, warn_total = get_grouped_event_totals(df)

if crit_total >= 1:

return score, "NOT READY FOR FLIGHT"

elif warn_total >= 1:

return score, "MAINTENANCE REQUIRED"

else:

return score, "READY FOR FLIGHT"

def generate_grouped_readiness_explanation(df: pd.DataFrame) -> List[str]:

explanations = []

priority_order = [

"egt_c", "engine_rpm_pct", "oil_pressure_psi", "oil_temp_c",

"vibration_g", "fuel_flow_kgph", "pitch_deg", "roll_deg", "airspeed_kts"

]

for col in priority_order:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

crit_count = int((grouped["severity"] == "Critical").sum())

warn_count = int((grouped["severity"] == "Warning").sum())

if crit_count > 0:

explanations.append(f"{RULES[col].display_name} has {crit_count} critical incident(s).")

elif warn_count > 0:

explanations.append(f"{RULES[col].display_name} has {warn_count} warning incident(s).")

if not explanations:

explanations.append("No major grouped incidents identified.")

return explanations[:6]

def build_root_cause_summary(df: pd.DataFrame) -> pd.DataFrame:

rows = []

for col in RULES:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

crit = grouped[grouped["severity"] == "Critical"]

warn = grouped[grouped["severity"] == "Warning"]

rows.append({

"Parameter": RULES[col].display_name,

"Group": RULES[col].group,

"Critical Incidents": len(crit),

"Warning Incidents": len(warn),

"Total Incidents": len(grouped),

"Peak Value": round(float(grouped["peak_value"].max()), 2),

"Unit": RULES[col].unit,

})

if not rows:

return pd.DataFrame()

return pd.DataFrame(rows).sort_values(

["Critical Incidents", "Warning Incidents", "Total Incidents", "Peak Value"],

ascending=[False, False, False, False],

).reset_index(drop=True)

def rank_root_causes(df: pd.DataFrame) -> pd.DataFrame:

rows = []

for col in RULES:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

crit = grouped[grouped["severity"] == "Critical"]

warn = grouped[grouped["severity"] == "Warning"]

impact = len(crit) * 5 + len(warn) * 2 + grouped["duration_sec"].sum() * 0.1

rows.append({

"Parameter": RULES[col].display_name,

"Critical": len(crit),

"Warning": len(warn),

"Total Incidents": len(grouped),

"Impact Score": round(float(impact), 2),

})

if not rows:

return pd.DataFrame()

return pd.DataFrame(rows).sort_values("Impact Score", ascending=False).reset_index(drop=True)

# ---------------------------

# Explanations and recommendations

# ---------------------------

def explain_spike(df: pd.DataFrame, idx: int, col: str) -> str:

row = df.loc[idx]

phase = row["phase"] if "phase" in df.columns else "unknown phase"

anomaly = row["anomaly_type"] if "anomaly_type" in df.columns else "none"

rpm = row["engine_rpm_pct"] if "engine_rpm_pct" in df.columns else np.nan

egt = row["egt_c"] if "egt_c" in df.columns else np.nan

fuel = row["fuel_flow_kgph"] if "fuel_flow_kgph" in df.columns else np.nan

oil_t = row["oil_temp_c"] if "oil_temp_c" in df.columns else np.nan

vib = row["vibration_g"] if "vibration_g" in df.columns else np.nan

pitch = row["pitch_deg"] if "pitch_deg" in df.columns else np.nan

roll = row["roll_deg"] if "roll_deg" in df.columns else np.nan

reasons = []

if str(anomaly).lower() not in ["none", "nan", ""]:

reasons.append(f"dataset anomaly tag: {anomaly}")

if col == "egt_c":

if pd.notna(rpm) and rpm > 100:

reasons.append("engine RPM is also high, so the EGT spike is likely linked to a high-power engine condition")

if pd.notna(fuel) and fuel > 6500:

reasons.append("fuel flow is elevated, suggesting increased combustion demand")

if pd.notna(oil_t) and oil_t > 130:

reasons.append("oil temperature is also high, indicating broader engine thermal stress")

elif col == "engine_rpm_pct":

if pd.notna(egt) and egt > 780:

reasons.append("EGT is high at the same time, which supports an engine overspeed or thermal load event")

if pd.notna(fuel) and fuel > 6500:

reasons.append("fuel flow increased with RPM, which matches a thrust-demand surge")

elif col == "fuel_flow_kgph":

if pd.notna(rpm) and rpm > 100:

reasons.append("engine RPM is high, so fuel flow rose with engine demand")

if pd.notna(egt) and egt > 780:

reasons.append("higher fuel burn appears to be contributing to the temperature spike")

elif col == "oil_pressure_psi":

if pd.notna(oil_t) and oil_t > 130:

reasons.append("oil pressure dropped while oil temperature was elevated, suggesting lubrication stress")

if pd.notna(vib) and vib > 1.8:

reasons.append("higher vibration may be affecting lubrication stability")

elif col == "oil_temp_c":

if pd.notna(egt) and egt > 780:

reasons.append("oil temperature rose alongside engine exhaust heat")

if pd.notna(rpm) and rpm > 100:

reasons.append("sustained high RPM can increase oil heating")

elif col == "vibration_g":

if pd.notna(rpm) and rpm > 100:

reasons.append("high engine RPM can amplify mechanical vibration")

if pd.notna(roll) and abs(roll) > 60:

reasons.append("aggressive maneuvering may be contributing to vibration")

elif col in ["pitch_deg", "roll_deg", "yaw_deg"]:

if phase not in ["cruise", "unknown phase"]:

reasons.append(f"the aircraft is in {phase}, so attitude change may be due to maneuvering")

if pd.notna(vib) and vib > 1.8:

reasons.append("vibration is elevated during the attitude excursion")

elif col == "altitude_ft":

if phase in ["descent", "landing", "approach"]:

reasons.append("altitude change is consistent with descent or landing phase")

if pd.notna(pitch) and pitch < -10:

reasons.append("nose-down pitch suggests a strong descent profile")

elif col == "airspeed_kts":

if phase in ["takeoff", "climb", "descent"]:

reasons.append(f"airspeed changed during {phase}, which can naturally create a spike")

if pd.notna(pitch) and abs(pitch) > 15:

reasons.append("pitch excursion may be affecting airspeed behavior")

if not reasons:

reasons.append("this point crossed the configured threshold for the selected Category II parameter")

return "; ".join(reasons).capitalize() + "."

def get_flagged_points(df: pd.DataFrame, col: str) -> pd.DataFrame:

sev = classify_series(df[col], RULES[col])

flagged = df.loc[sev.isin(["Warning", "Critical"]), ["timestamp", "phase", col]].copy()

flagged["severity"] = sev.loc[flagged.index].values

flagged["explanation"] = [explain_spike(df, i, col) for i in flagged.index]

return flagged.sort_values(["severity", col], ascending=[False, False]).head(15)

def recommendation_text(df: pd.DataFrame) -> List[str]:

recs = []

for col in ["egt_c", "engine_rpm_pct", "oil_pressure_psi", "oil_temp_c", "vibration_g", "fuel_flow_kgph"]:

if col not in df.columns:

continue

sev = classify_series(df[col], RULES[col])

if (sev == "Critical").any():

if col == "egt_c":

recs.append("Inspect turbine or hot-section condition and verify throttle-to-temperature behavior.")

elif col == "engine_rpm_pct":

recs.append("Check engine control, governor response, and overspeed logic.")

elif col == "oil_pressure_psi":

recs.append("Inspect lubrication system, oil pump, and line pressure stability.")

elif col == "oil_temp_c":

recs.append("Check cooling efficiency and prolonged thermal loading on the oil circuit.")

elif col == "vibration_g":

recs.append("Review rotating components and maneuver-linked vibration patterns.")

elif col == "fuel_flow_kgph":

recs.append("Validate abnormal fuel demand against RPM, EGT, and flight phase.")

if not recs:

recs.append("No urgent engine-health recommendation generated from current thresholds.")

return recs[:6]

def build_report_dataframe(df: pd.DataFrame) -> pd.DataFrame:

analytics_score, _ = readiness_score(df)

readiness_score_value, readiness_status = get_readiness_status(df)

crit_total, warn_total = get_grouped_event_totals(df)

root_cause_df = build_root_cause_summary(df)

ranked_df = rank_root_causes(df)

recommendations = recommendation_text(df)

explanations = generate_grouped_readiness_explanation(df)

rows = [

{"Section": "Summary", "Item": "Flight Status", "Value": readiness_status},

{"Section": "Summary", "Item": "Analytics Score", "Value": analytics_score},

{"Section": "Summary", "Item": "Operational Readiness Score", "Value": readiness_score_value},

{"Section": "Summary", "Item": "Critical Incidents", "Value": crit_total},

{"Section": "Summary", "Item": "Warning Incidents", "Value": warn_total},

]

for i, text in enumerate(explanations, start=1):

rows.append({"Section": "Why Not Ready", "Item": f"Explanation {i}", "Value": text})

if not ranked_df.empty:

for i, (_, row) in enumerate(ranked_df.head(3).iterrows(), start=1):

rows.append({

"Section": "Root Cause Ranking",

"Item": f"Cause {i}",

"Value": f"{row['Parameter']} | Critical={row['Critical']} | Warning={row['Warning']} | Impact Score={row['Impact Score']}"

})

if not root_cause_df.empty:

for i, (_, row) in enumerate(root_cause_df.head(5).iterrows(), start=1):

rows.append({

"Section": "Root Cause Summary",

"Item": f"Parameter {i}",

"Value": f"{row['Parameter']} | Critical Incidents={row['Critical Incidents']} | Warning Incidents={row['Warning Incidents']} | Peak Value={row['Peak Value']} {row['Unit']}"

})

for i, text in enumerate(recommendations, start=1):

rows.append({"Section": "Recommendations", "Item": f"Recommendation {i}", "Value": text})

return pd.DataFrame(rows)

# ---------------------------

# Charts

# ---------------------------

def create_line_chart(df: pd.DataFrame, col: str, title_prefix: str = ""):

rule = RULES[col]

plot_df = df[["timestamp", "phase", col]].dropna().copy()

plot_df["severity"] = classify_series(plot_df[col], rule)

plot_df["hover_note"] = [

explain_spike(df, i, col) if plot_df.loc[i, "severity"] != "Normal" else "Normal operating region."

for i in plot_df.index

]

fig = go.Figure()

fig.add_trace(go.Scatter(

x=plot_df["timestamp"],

y=plot_df[col],

mode="lines",

name=rule.display_name,

line=dict(width=2),

))

flagged = plot_df[plot_df["severity"].isin(["Warning", "Critical"])]

if not flagged.empty:

fig.add_trace(go.Scatter(

x=flagged["timestamp"],

y=flagged[col],

mode="markers",

name="Flagged points",

marker=dict(size=9, symbol="diamond"),

text=flagged["hover_note"],

hovertemplate="Time=%{x}<br>Value=%{y}<br>%{text}<extra></extra>",

))

if rule.warn_high is not None:

fig.add_hline(y=rule.warn_high, line_dash="dash", annotation_text="Warn High")

if rule.warn_low is not None:

fig.add_hline(y=rule.warn_low, line_dash="dash", annotation_text="Warn Low")

if rule.crit_high is not None:

fig.add_hline(y=rule.crit_high, line_dash="dot", annotation_text="Critical High")

if rule.crit_low is not None:

fig.add_hline(y=rule.crit_low, line_dash="dot", annotation_text="Critical Low")

title = f"{title_prefix} - {rule.display_name}" if title_prefix else rule.display_name

fig.update_layout(height=430, title=title, xaxis_title="Timestamp", yaxis_title=f"{rule.display_name} ({rule.unit})")

st.plotly_chart(fig, use_container_width=True)

def create_phase_boxplot(df: pd.DataFrame, col: str):

rule = RULES[col]

plot_df = df[["phase", col]].dropna().copy()

if plot_df.empty:

return

fig = px.box(plot_df, x="phase", y=col, color="phase", title=f"{rule.display_name} by Flight Phase")

fig.update_layout(height=380, xaxis_title="Phase", yaxis_title=f"{rule.display_name} ({rule.unit})")

st.plotly_chart(fig, use_container_width=True)

def create_histogram(df: pd.DataFrame, col: str):

rule = RULES[col]

plot_df = df[[col]].dropna().copy()

if plot_df.empty:

return

fig = px.histogram(plot_df, x=col, nbins=40, title=f"Distribution of {rule.display_name}")

fig.update_layout(height=360, xaxis_title=f"{rule.display_name} ({rule.unit})")

st.plotly_chart(fig, use_container_width=True)

def create_correlation_heatmap(df: pd.DataFrame):

numeric_cols = [c for c in RULES if c in df.columns]

if len(numeric_cols) < 2:

st.info("Not enough numeric columns for correlation view.")

return

corr = df[numeric_cols].corr(numeric_only=True)

fig = px.imshow(corr, text_auto=True, aspect="auto", title="Correlation Heatmap of Category II Parameters")

fig.update_layout(height=650)

st.plotly_chart(fig, use_container_width=True)

def create_status_bar(df: pd.DataFrame):

rows = []

for col in RULES:

if col not in df.columns:

continue

sev = classify_series(df[col], RULES[col])

rows.append({

"Parameter": RULES[col].display_name,

"Normal": int((sev == "Normal").sum()),

"Warning": int((sev == "Warning").sum()),

"Critical": int((sev == "Critical").sum()),

})

status_df = pd.DataFrame(rows)

if status_df.empty:

return

melted = status_df.melt(id_vars="Parameter", var_name="Status", value_name="Count")

fig = px.bar(melted, x="Parameter", y="Count", color="Status", title="Parameter-wise Event Severity Count", barmode="stack")

fig.update_layout(height=450, xaxis_tickangle=-45)

st.plotly_chart(fig, use_container_width=True)

def create_anomaly_by_phase(df: pd.DataFrame):

if "anomaly_type" not in df.columns or "phase" not in df.columns:

return

temp = df.groupby(["phase", "anomaly_type"], as_index=False).size().rename(columns={"size": "count"})

fig = px.bar(temp, x="phase", y="count", color="anomaly_type", title="Anomaly Type Distribution by Flight Phase", barmode="group")

fig.update_layout(height=420)

st.plotly_chart(fig, use_container_width=True)

def create_kpi_cards(df: pd.DataFrame):

analytics_score, notes = readiness_score(df)

readiness_score_value, readiness_status = get_readiness_status(df)

crit_total, warn_total = get_grouped_event_totals(df)

c1, c2, c3, c4 = st.columns(4)

with c1:

st.metric("Analytics Score", analytics_score)

with c2:

st.metric("Flight Status", readiness_status)

with c3:

st.metric("Critical Incidents", crit_total)

with c4:

st.metric("Warning Incidents", warn_total)

st.markdown("### Flight Readiness Status")

if readiness_status == "READY FOR FLIGHT":

st.success(f"Flight Status: {readiness_status} | Operational Readiness Score: {readiness_score_value}")

elif readiness_status == "MAINTENANCE REQUIRED":

st.warning(f"Flight Status: {readiness_status} | Operational Readiness Score: {readiness_score_value}")

else:

st.error(f"Flight Status: {readiness_status} | Operational Readiness Score: {readiness_score_value}")

if readiness_status != "READY FOR FLIGHT":

st.markdown("### Why is the aircraft not ready?")

for reason in generate_grouped_readiness_explanation(df):

st.write(f"- {reason}")

root_cause_df = build_root_cause_summary(df)

if not root_cause_df.empty:

st.markdown("### Top grouped root causes")

st.dataframe(root_cause_df.head(5), use_container_width=True)

ranked = rank_root_causes(df)

if not ranked.empty:

st.markdown("### Root Cause Ranking")

top3 = ranked.head(3)

for i, row in top3.iterrows():

label = "Primary Cause" if i == 0 else f"Secondary Cause {i}"

st.write(f"{label}: {row['Parameter']} (Critical: {row['Critical']}, Warning: {row['Warning']})")

st.markdown("### Parameter-Level Explanation")

if notes:

for note in notes:

st.write(f"- {note}")

else:

st.write("- No repeated threshold exceedance pattern was detected in the main parameters.")

# ---------------------------

# UI

# ---------------------------

st.title("✈️ Fighter Aircraft Flight Readiness System")

st.caption("Category II – Fighter / Attack / Trainer")

with st.sidebar:

st.header("Upload Dataset")

uploaded = st.file_uploader("Upload fighter aircraft CSV", type=["csv", "xlsx"])

search_text = st.text_input("Search button input", placeholder="EGT, RPM, fuel, pitch, altitude...")

run_search = st.button("Search")

st.markdown("---")

st.write("This app is tuned for the uploaded synthetic telemetry format.")

if uploaded is None:

st.info("Upload your CSV file to start.")

st.code(

"""

Expected sample columns:

- timestamp

- phase

- airspeed_kts

- altitude_ft

- pitch_deg

- roll_deg

- yaw_deg

- engine_rpm_pct

- egt_c

- fuel_flow_kgph

- oil_pressure_psi

- oil_temp_c

- vibration_g

- ambient_temp_c

- anomaly_type

""".strip()

)

st.stop()

if uploaded.name.lower().endswith(".csv"):

df = pd.read_csv(uploaded)

else:

df = pd.read_excel(uploaded)

df = preprocess(df)

create_kpi_cards(df)

st.markdown("### Recommendations")

for item in recommendation_text(df):

st.write(f"- {item}")

report_df = build_report_dataframe(df)

csv_data = report_df.to_csv(index=False).encode("utf-8")

st.download_button(

label="Download Incident Report (CSV)",

data=csv_data,

file_name="flight_readiness_incident_report.csv",

mime="text/csv",

)

if run_search and search_text:

chosen = search_parameter(search_text, df.columns.tolist())

st.markdown("---")

st.markdown("## Search Result")

if chosen is None:

st.warning("No matching column or domain found for the search text.")

else:

create_line_chart(df, chosen, "Search")

flagged = get_flagged_points(df, chosen)

if flagged.empty:

st.success("No threshold spikes detected for the searched parameter.")

else:

show_cols = ["timestamp", "phase", chosen, "severity", "explanation"]

renamed = flagged[show_cols].rename(columns={chosen: f"Value ({RULES[chosen].unit})"})

st.dataframe(renamed, use_container_width=True)

st.markdown("---")

tab1, tab2, tab3, tab4, tab5, tab6, tab7, tab8 = st.tabs([

"Overview",

"Engine Health",

"Flight Attitude",

"Phase Analysis",

"Search Any Parameter",

"Incident Timeline",

"Root Cause Summary",

"Data Preview",

])

with tab1:

create_status_bar(df)

c1, c2 = st.columns(2)

with c1:

if "engine_rpm_pct" in df.columns:

create_histogram(df, "engine_rpm_pct")

with c2:

if "egt_c" in df.columns:

create_histogram(df, "egt_c")

create_correlation_heatmap(df)

create_anomaly_by_phase(df)

with tab2:

engine_choices = [c for c in ["engine_rpm_pct", "egt_c", "fuel_flow_kgph", "oil_pressure_psi", "oil_temp_c", "vibration_g"] if c in df.columns]

selected_engine = st.multiselect(

"Choose engine graphs",

engine_choices,

default=engine_choices[: min(3, len(engine_choices))],

format_func=lambda x: RULES[x].display_name,

key="engine_multiselect",

)

for col in selected_engine:

create_line_chart(df, col, "Engine Health")

flagged = get_flagged_points(df, col)

if not flagged.empty:

st.dataframe(

flagged[["timestamp", "phase", col, "severity", "explanation"]].rename(columns={col: f"Value ({RULES[col].unit})"}),

use_container_width=True,

)

with tab3:

attitude_choices = [c for c in ["pitch_deg", "roll_deg", "yaw_deg", "airspeed_kts", "altitude_ft"] if c in df.columns]

if attitude_choices:

selected_att = st.selectbox(

"Choose flight attitude or profile parameter",

attitude_choices,

format_func=lambda x: RULES[x].display_name,

key="attitude_select",

)

create_line_chart(df, selected_att, "Flight")

c1, c2 = st.columns(2)

with c1:

create_phase_boxplot(df, selected_att)

with c2:

create_histogram(df, selected_att)

flagged = get_flagged_points(df, selected_att)

if not flagged.empty:

st.dataframe(

flagged[["timestamp", "phase", selected_att, "severity", "explanation"]].rename(columns={selected_att: f"Value ({RULES[selected_att].unit})"}),

use_container_width=True,

)

with tab4:

phase_metrics = [c for c in RULES if c in df.columns]

if phase_metrics:

selected_phase_metric = st.selectbox(

"Choose parameter for phase comparison",

phase_metrics,

format_func=lambda x: RULES[x].display_name,

key="phase_metric_select",

)

create_phase_boxplot(df, selected_phase_metric)

create_line_chart(df, selected_phase_metric, "Phase Trend")

with tab5:

typed = st.text_input("Type a column name or domain", value="egt", key="tab_search")

chosen = search_parameter(typed, df.columns.tolist())

if chosen is None:

st.warning("No matching parameter found.")

else:

create_line_chart(df, chosen, "Dynamic Search")

flagged = get_flagged_points(df, chosen)

if flagged.empty:

st.success("No threshold spikes detected.")

else:

st.dataframe(

flagged[["timestamp", "phase", chosen, "severity", "explanation"]].rename(columns={chosen: f"Value ({RULES[chosen].unit})"}),

use_container_width=True,

)

with tab6:

st.markdown("### Grouped Incident Timeline")

incident_cols = [c for c in RULES if c in df.columns]

if not incident_cols:

st.info("No valid parameters found for incident analysis.")

else:

selected_incident_col = st.selectbox(

"Choose parameter",

incident_cols,

format_func=lambda x: RULES[x].display_name,

)

grouped = group_consecutive_events(df, selected_incident_col)

if grouped.empty:

st.success("No warning or critical incidents found.")

else:

st.dataframe(grouped, use_container_width=True)

with tab7:

st.markdown("### Root Cause Summary")

root_cause_df = build_root_cause_summary(df)

if root_cause_df.empty:

st.success("No grouped incidents available for root-cause summary.")

else:

st.dataframe(root_cause_df, use_container_width=True)

with tab8:

st.write("Detected columns:", list(df.columns))

st.dataframe(df.head(50), use_container_width=True)