import re

from dataclasses import dataclass

from typing import Dict, List, Optional, Tuple

import numpy as np

import pandas as pd

import plotly.express as px

import plotly.graph_objects as go

import streamlit as st

st.set_page_config(page_title="Fighter Aircraft Flight Readiness System", layout="wide")

# ============================================================

# Fighter Aircraft Flight Readiness System (Category II)

# Stable demo version for the uploaded telemetry dataset

# ============================================================

# Expected dataset columns:

# timestamp, phase, airspeed_kts, altitude_ft, pitch_deg, roll_deg, yaw_deg,

# engine_rpm_pct, egt_c, fuel_flow_kgph, oil_pressure_psi, oil_temp_c,

# vibration_g, ambient_temp_c, anomaly_type

# ---------------------------

# Parameter rules

# ---------------------------

@dataclass

class ParameterRule:

key: str

display_name: str

group: str

unit: str

warn_low: Optional[float] = None

warn_high: Optional[float] = None

crit_low: Optional[float] = None

crit_high: Optional[float] = None

description: str = ""

RULES: Dict[str, ParameterRule] = {

"airspeed_kts": ParameterRule("airspeed_kts", "Airspeed", "Flight Profile", "kts", warn_low=20, warn_high=650, crit_low=0, crit_high=750, description="Aircraft speed envelope"),

"altitude_ft": ParameterRule("altitude_ft", "Altitude", "Flight Profile", "ft", warn_low=100, crit_low=50, description="Altitude trend"),

"pitch_deg": ParameterRule("pitch_deg", "Pitch", "Flight Attitude", "deg", warn_low=-25, warn_high=25, crit_low=-35, crit_high=35, description="Nose up/down attitude"),

"roll_deg": ParameterRule("roll_deg", "Roll", "Flight Attitude", "deg", warn_low=-60, warn_high=60, crit_low=-80, crit_high=80, description="Bank angle"),

"yaw_deg": ParameterRule("yaw_deg", "Yaw", "Flight Attitude", "deg", warn_low=-25, warn_high=25, crit_low=-35, crit_high=35, description="Yaw angle"),

"engine_rpm_pct": ParameterRule("engine_rpm_pct", "Engine RPM", "Engine Health", "%", warn_high=100, crit_high=106, description="Engine spool speed"),

"egt_c": ParameterRule("egt_c", "Exhaust Gas Temperature", "Engine Health", "°C", warn_high=780, crit_high=823, description="Exhaust thermal loading"),

"fuel_flow_kgph": ParameterRule("fuel_flow_kgph", "Fuel Flow", "Engine Health", "kg/h", warn_high=6500, crit_high=7500, description="Fuel consumption rate"),

"oil_pressure_psi": ParameterRule("oil_pressure_psi", "Oil Pressure", "Engine Health", "psi", warn_low=30, crit_low=25.7, description="Engine lubrication pressure"),

"oil_temp_c": ParameterRule("oil_temp_c", "Oil Temperature", "Engine Health", "°C", warn_high=130, crit_high=147, description="Oil system temperature"),

"vibration_g": ParameterRule("vibration_g", "Engine Vibration", "Engine Health", "g", warn_high=1.8, crit_high=2.4, description="Engine / airframe vibration level"),

"ambient_temp_c": ParameterRule("ambient_temp_c", "Ambient Temperature", "Environment", "°C", warn_low=-30, warn_high=45, crit_low=-40, crit_high=55, description="Outside air conditions"),

}

SEARCH_MAP = {

"airspeed": "airspeed_kts",

"speed": "airspeed_kts",

"altitude": "altitude_ft",

"height": "altitude_ft",

"pitch": "pitch_deg",

"roll": "roll_deg",

"yaw": "yaw_deg",

"rpm": "engine_rpm_pct",

"engine": "engine_rpm_pct",

"egt": "egt_c",

"temperature": "egt_c",

"engine temperature": "egt_c",

"fuel": "fuel_flow_kgph",

"fuel flow": "fuel_flow_kgph",

"oil pressure": "oil_pressure_psi",

"oil": "oil_pressure_psi",

"oil temp": "oil_temp_c",

"oil temperature": "oil_temp_c",

"vibration": "vibration_g",

"ambient": "ambient_temp_c",

"ambient temperature": "ambient_temp_c",

}

# ---------------------------

# Utilities

# ---------------------------

def normalize_text(text: str) -> str:

return re.sub(r"[^a-z0-9]+", " ", str(text).lower()).strip()

def preprocess(df: pd.DataFrame) -> pd.DataFrame:

df = df.copy()

df["timestamp"] = pd.to_datetime(df["timestamp"], errors="coerce")

df = df.sort_values("timestamp").reset_index(drop=True)

df["time_index"] = np.arange(len(df))

for col in RULES:

if col in df.columns:

df[col] = pd.to_numeric(df[col], errors="coerce")

if "phase" in df.columns:

df["phase"] = df["phase"].astype(str)

else:

df["phase"] = "unknown"

if "anomaly_type" in df.columns:

df["anomaly_type"] = df["anomaly_type"].astype(str)

else:

df["anomaly_type"] = "none"

return df

def classify_series(series: pd.Series, rule: ParameterRule) -> pd.Series:

sev = pd.Series("Normal", index=series.index)

if rule.warn_low is not None:

sev.loc[series < rule.warn_low] = "Warning"

if rule.warn_high is not None:

sev.loc[series > rule.warn_high] = "Warning"

if rule.crit_low is not None:

sev.loc[series < rule.crit_low] = "Critical"

if rule.crit_high is not None:

sev.loc[series > rule.crit_high] = "Critical"

return sev

def search_parameter(user_text: str, available_cols: List[str]) -> Optional[str]:

user_text = normalize_text(user_text)

if not user_text:

return None

for key, mapped in SEARCH_MAP.items():

if key in user_text and mapped in available_cols:

return mapped

for col in available_cols:

if normalize_text(col) in user_text or user_text in normalize_text(col):

return col

for col in available_cols:

rule = RULES.get(col)

if rule and user_text in normalize_text(rule.display_name + " " + rule.group + " " + rule.description):

return col

return None

def parse_value_parameter_search(user_text: str, available_cols: List[str]) -> Tuple[Optional[float], Optional[str]]:

raw = str(user_text).strip()

if not raw:

return None, None

parts = [p.strip() for p in raw.split(",") if p.strip()]

target_value = None

parameter_text = None

if len(parts) >= 2:

try:

target_value = float(parts[0])

parameter_text = ",".join(parts[1:])

except ValueError:

parameter_text = raw

else:

if re.fullmatch(r"[-+]?\d*\.?\d+", raw):

target_value = float(raw)

else:

parameter_text = raw

chosen_col = search_parameter(parameter_text, available_cols) if parameter_text else None

return target_value, chosen_col

def find_value_hit_rows(df: pd.DataFrame, search_col: str, target_value: float, max_hits: int = 5) -> pd.DataFrame:

if search_col not in df.columns:

return pd.DataFrame()

work = df[["timestamp", "phase", search_col]].copy().dropna()

if work.empty:

return pd.DataFrame()

work["abs_diff"] = (work[search_col] - target_value).abs()

exact = work[np.isclose(work[search_col], target_value, atol=1e-9)]

if not exact.empty:

result = exact.sort_values("timestamp").head(max_hits).copy()

result["match_type"] = "Exact"

return result

closest = work.sort_values(["abs_diff", "timestamp"]).head(max_hits).copy()

closest["match_type"] = "Closest"

return closest

def create_event_context_charts(

df: pd.DataFrame,

trigger_col: str,

target_value: float,

hit_rows: pd.DataFrame,

window_size: int = 20,

):

if hit_rows.empty:

st.warning("No matching or nearby rows found for the requested value search.")

return

st.markdown("### Value-hit search summary")

summary = hit_rows[["timestamp", "phase", trigger_col, "abs_diff", "match_type"]].rename(

columns={

trigger_col: f"{RULES[trigger_col].display_name} ({RULES[trigger_col].unit})",

"abs_diff": "Difference from searched value",

"match_type": "Match Type",

}

)

st.dataframe(summary, use_container_width=True)

event_indices = hit_rows.index.tolist()

keep_indices = set()

for idx in event_indices:

start = max(0, idx - window_size)

end = min(len(df) - 1, idx + window_size)

keep_indices.update(range(start, end + 1))

context_df = df.loc[sorted(keep_indices)].copy()

available_plot_cols = [c for c in RULES if c in context_df.columns]

st.markdown(

f"### All parameter trends around {RULES[trigger_col].display_name} hitting {target_value}"

)

for col in available_plot_cols:

rule = RULES[col]

fig = go.Figure()

fig.add_trace(

go.Scatter(

x=context_df["timestamp"],

y=context_df[col],

mode="lines",

name=rule.display_name,

line=dict(width=2),

)

)

hit_points = df.loc[event_indices, ["timestamp", col]].dropna()

if not hit_points.empty:

fig.add_trace(

go.Scatter(

x=hit_points["timestamp"],

y=hit_points[col],

mode="markers",

name="Value-hit points",

marker=dict(size=10, symbol="diamond"),

hovertemplate="Time=%{x}<br>Value=%{y}<extra></extra>",

)

)

for ts in hit_points["timestamp"]:

fig.add_vline(x=ts, line_dash="dot")

if col == trigger_col:

fig.add_hline(

y=target_value,

line_dash="dash",

annotation_text=f"Searched Value = {target_value}",

)

if rule.warn_high is not None:

fig.add_hline(y=rule.warn_high, line_dash="dash", annotation_text="Warn High")

if rule.warn_low is not None:

fig.add_hline(y=rule.warn_low, line_dash="dash", annotation_text="Warn Low")

if rule.crit_high is not None:

fig.add_hline(y=rule.crit_high, line_dash="dot", annotation_text="Critical High")

if rule.crit_low is not None:

fig.add_hline(y=rule.crit_low, line_dash="dot", annotation_text="Critical Low")

fig.update_layout(

height=340,

title=f"{rule.display_name} around {RULES[trigger_col].display_name} = {target_value}",

xaxis_title="Timestamp",

yaxis_title=f"{rule.display_name} ({rule.unit})",

)

st.plotly_chart(fig, use_container_width=True)

# ---------------------------

# Event grouping and readiness

# ---------------------------

def group_consecutive_events(df: pd.DataFrame, col: str) -> pd.DataFrame:

if col not in df.columns:

return pd.DataFrame()

rule = RULES[col]

sev = classify_series(df[col], rule)

work = df[["timestamp", "phase", col]].copy()

work["severity"] = sev

work = work[work["severity"].isin(["Warning", "Critical"])].copy()

if work.empty:

return pd.DataFrame()

work["event_break"] = (

(work["severity"] != work["severity"].shift(1)) |

((work.index.to_series() - work.index.to_series().shift(1)) > 1)

)

work["event_id"] = work["event_break"].cumsum()

grouped = (

work.groupby("event_id", as_index=False)

.agg(

parameter=(col, lambda _: RULES[col].display_name),

severity=("severity", "first"),

start_time=("timestamp", "first"),

end_time=("timestamp", "last"),

phase=("phase", "first"),

peak_value=(col, "max"),

min_value=(col, "min"),

sample_count=(col, "size"),

)

)

grouped["duration_sec"] = (

grouped["end_time"] - grouped["start_time"]

).dt.total_seconds().fillna(0)

grouped["unit"] = RULES[col].unit

return grouped

def get_grouped_event_totals(df: pd.DataFrame) -> Tuple[int, int]:

critical_events = 0

warning_events = 0

for col in RULES:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

critical_events += int((grouped["severity"] == "Critical").sum())

warning_events += int((grouped["severity"] == "Warning").sum())

return critical_events, warning_events

def readiness_score(df: pd.DataFrame) -> Tuple[float, List[str]]:

penalties = 0.0

notes = []

important = [

"engine_rpm_pct", "egt_c", "fuel_flow_kgph", "oil_pressure_psi",

"oil_temp_c", "vibration_g", "pitch_deg", "roll_deg", "airspeed_kts"

]

for col in important:

if col not in df.columns:

continue

sev = classify_series(df[col], RULES[col])

warn_rate = (sev == "Warning").mean()

crit_rate = (sev == "Critical").mean()

penalties += warn_rate * 10 + crit_rate * 25

if crit_rate > 0.01:

notes.append(f"{RULES[col].display_name} has repeated critical excursions.")

elif warn_rate > 0.03:

notes.append(f"{RULES[col].display_name} shows warning-level deviations.")

score = max(0, round(100 - penalties, 1))

return score, notes[:6]

def get_readiness_status(df: pd.DataFrame) -> Tuple[float, str]:

score, _ = readiness_score(df)

crit_total, warn_total = get_grouped_event_totals(df)

if crit_total >= 1:

return score, "NOT READY FOR FLIGHT"

elif warn_total >= 1:

return score, "MAINTENANCE REQUIRED"

else:

return score, "READY FOR FLIGHT"

def generate_grouped_readiness_explanation(df: pd.DataFrame) -> List[str]:

explanations = []

priority_order = [

"egt_c", "engine_rpm_pct", "oil_pressure_psi", "oil_temp_c",

"vibration_g", "fuel_flow_kgph", "pitch_deg", "roll_deg", "airspeed_kts"

]

for col in priority_order:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

crit_count = int((grouped["severity"] == "Critical").sum())

warn_count = int((grouped["severity"] == "Warning").sum())

if crit_count > 0:

explanations.append(f"{RULES[col].display_name} has {crit_count} critical incident(s).")

elif warn_count > 0:

explanations.append(f"{RULES[col].display_name} has {warn_count} warning incident(s).")

if not explanations:

explanations.append("No major grouped incidents identified.")

return explanations[:6]

def build_root_cause_summary(df: pd.DataFrame) -> pd.DataFrame:

rows = []

for col in RULES:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

crit = grouped[grouped["severity"] == "Critical"]

warn = grouped[grouped["severity"] == "Warning"]

rows.append({

"Parameter": RULES[col].display_name,

"Group": RULES[col].group,

"Critical Incidents": len(crit),

"Warning Incidents": len(warn),

"Total Incidents": len(grouped),

"Peak Value": round(float(grouped["peak_value"].max()), 2),

"Unit": RULES[col].unit,

})

if not rows:

return pd.DataFrame()

return pd.DataFrame(rows).sort_values(

["Critical Incidents", "Warning Incidents", "Total Incidents", "Peak Value"],

ascending=[False, False, False, False],

).reset_index(drop=True)

def rank_root_causes(df: pd.DataFrame) -> pd.DataFrame:

rows = []

for col in RULES:

if col not in df.columns:

continue

grouped = group_consecutive_events(df, col)

if grouped.empty:

continue

grouped = grouped[grouped["sample_count"] >= 3].copy()

if grouped.empty:

continue

crit = grouped[grouped["severity"] == "Critical"]

warn = grouped[grouped["severity"] == "Warning"]

impact = len(crit) * 5 + len(warn) * 2 + grouped["duration_sec"].sum() * 0.1

rows.append({

"Parameter": RULES[col].display_name,

"Critical": len(crit),

"Warning": len(warn),

"Total Incidents": len(grouped),

"Impact Score": round(float(impact), 2),

})

if not rows:

return pd.DataFrame()

return pd.DataFrame(rows).sort_values("Impact Score", ascending=False).reset_index(drop=True)

# ---------------------------

# Explanations and recommendations

# ---------------------------

def explain_spike(df: pd.DataFrame, idx: int, col: str) -> str:

row = df.loc[idx]

phase = row["phase"] if "phase" in df.columns else "unknown phase"

anomaly = row["anomaly_type"] if "anomaly_type" in df.columns else "none"

rpm = row["engine_rpm_pct"] if "engine_rpm_pct" in df.columns else np.nan

egt = row["egt_c"] if "egt_c" in df.columns else np.nan

fuel = row["fuel_flow_kgph"] if "fuel_flow_kgph" in df.columns else np.nan

oil_t = row["oil_temp_c"] if "oil_temp_c" in df.columns else np.nan

vib = row["vibration_g"] if "vibration_g" in df.columns else np.nan

pitch = row["pitch_deg"] if "pitch_deg" in df.columns else np.nan

roll = row["roll_deg"] if "roll_deg" in df.columns else np.nan

reasons = []

if str(anomaly).lower() not in ["none", "nan", ""]:

reasons.append(f"dataset anomaly tag: {anomaly}")

if col == "egt_c":

if pd.notna(rpm) and rpm > 100:

reasons.append("engine RPM is also high, so the EGT spike is likely linked to a high-power engine condition")

if pd.notna(fuel) and fuel > 6500:

reasons.append("fuel flow is elevated, suggesting increased combustion demand")

if pd.notna(oil_t) and oil_t > 130:

reasons.append("oil temperature is also high, indicating broader engine thermal stress")

elif col == "engine_rpm_pct":

if pd.notna(egt) and egt > 780:

reasons.append("EGT is high at the same time, which supports an engine overspeed or thermal load event")

if pd.notna(fuel) and fuel > 6500:

reasons.append("fuel flow increased with RPM, which matches a thrust-demand surge")

elif col == "fuel_flow_kgph":

if pd.notna(rpm) and rpm > 100:

reasons.append("engine RPM is high, so fuel flow rose with engine demand")

if pd.notna(egt) and egt > 780:

reasons.append("higher fuel burn appears to be contributing to the temperature spike")

elif col == "oil_pressure_psi":

if pd.notna(oil_t) and oil_t > 130:

reasons.append("oil pressure dropped while oil temperature was elevated, suggesting lubrication stress")

if pd.notna(vib) and vib > 1.8:

reasons.append("higher vibration may be affecting lubrication stability")

elif col == "oil_temp_c":

if pd.notna(egt) and egt > 780:

reasons.append("oil temperature rose alongside engine exhaust heat")

if pd.notna(rpm) and rpm > 100:

reasons.append("sustained high RPM can increase oil heating")

elif col == "vibration_g":

if pd.notna(rpm) and rpm > 100:

reasons.append("high engine RPM can amplify mechanical vibration")

if pd.notna(roll) and abs(roll) > 60:

reasons.append("aggressive maneuvering may be contributing to vibration")

elif col in ["pitch_deg", "roll_deg", "yaw_deg"]:

if phase not in ["cruise", "unknown phase"]:

reasons.append(f"the aircraft is in {phase}, so attitude change may be due to maneuvering")

if pd.notna(vib) and vib > 1.8:

reasons.append("vibration is elevated during the attitude excursion")

elif col == "altitude_ft":

if phase in ["descent", "landing", "approach"]:

reasons.append("altitude change is consistent with descent or landing phase")

if pd.notna(pitch) and pitch < -10:

reasons.append("nose-down pitch suggests a strong descent profile")

elif col == "airspeed_kts":

if phase in ["takeoff", "climb", "descent"]:

reasons.append(f"airspeed changed during {phase}, which can naturally create a spike")

if pd.notna(pitch) and abs(pitch) > 15:

reasons.append("pitch excursion may be affecting airspeed behavior")

if not reasons:

reasons.append("this point crossed the configured threshold for the selected Category II parameter")

return "; ".join(reasons).capitalize() + "."

def get_flagged_points(df: pd.DataFrame, col: str) -> pd.DataFrame:

sev = classify_series(df[col], RULES[col])

flagged = df.loc[sev.isin(["Warning", "Critical"]), ["timestamp", "phase", col]].copy()

flagged["severity"] = sev.loc[flagged.index].values

flagged["explanation"] = [explain_spike(df, i, col) for i in flagged.index]

return flagged.sort_values(["severity", col], ascending=[False, False]).head(15)

def recommendation_text(df: pd.DataFrame) -> List[str]:

recs = []

for col in ["egt_c", "engine_rpm_pct", "oil_pressure_psi", "oil_temp_c", "vibration_g", "fuel_flow_kgph"]:

if col not in df.columns:

continue

sev = classify_series(df[col], RULES[col])

if (sev == "Critical").any():

if col == "egt_c":

recs.append("Inspect turbine or hot-section condition and verify throttle-to-temperature behavior.")

elif col == "engine_rpm_pct":

recs.append("Check engine control, governor response, and overspeed logic.")

elif col == "oil_pressure_psi":

recs.append("Inspect lubrication system, oil pump, and line pressure stability.")

elif col == "oil_temp_c":

recs.append("Check cooling efficiency and prolonged thermal loading on the oil circuit.")

elif col == "vibration_g":

recs.append("Review rotating components and maneuver-linked vibration patterns.")

elif col == "fuel_flow_kgph":

recs.append("Validate abnormal fuel demand against RPM, EGT, and flight phase.")

if not recs:

recs.append("No urgent engine-health recommendation generated from current thresholds.")

return recs[:6]

def build_report_dataframe(df: pd.DataFrame) -> pd.DataFrame:

analytics_score, _ = readiness_score(df)

readiness_score_value, readiness_status = get_readiness_status(df)

crit_total, warn_total = get_grouped_event_totals(df)

root_cause_df = build_root_cause_summary(df)

ranked_df = rank_root_causes(df)

recommendations = recommendation_text(df)

explanations = generate_grouped_readiness_explanation(df)

rows = [

{"Section": "Summary", "Item": "Flight Status", "Value": readiness_status},

{"Section": "Summary", "Item": "Analytics Score", "Value": analytics_score},

{"Section": "Summary", "Item": "Operational Readiness Score", "Value": readiness_score_value},

{"Section": "Summary", "Item": "Critical Incidents", "Value": crit_total},

{"Section": "Summary", "Item": "Warning Incidents", "Value": warn_total},

]

for i, text in enumerate(explanations, start=1):

rows.append({"Section": "Why Not Ready", "Item": f"Explanation {i}", "Value": text})

if not ranked_df.empty:

for i, (_, row) in enumerate(ranked_df.head(3).iterrows(), start=1):

rows.append({

"Section": "Root Cause Ranking",

"Item": f"Cause {i}",

"Value": f"{row['Parameter']} | Critical={row['Critical']} | Warning={row['Warning']} | Impact Score={row['Impact Score']}"

})

if not root_cause_df.empty:

for i, (_, row) in enumerate(root_cause_df.head(5).iterrows(), start=1):

rows.append({

"Section": "Root Cause Summary",

"Item": f"Parameter {i}",

"Value": f"{row['Parameter']} | Critical Incidents={row['Critical Incidents']} | Warning Incidents={row['Warning Incidents']} | Peak Value={row['Peak Value']} {row['Unit']}"

})

for i, text in enumerate(recommendations, start=1):

rows.append({"Section": "Recommendations", "Item": f"Recommendation {i}", "Value": text})

return pd.DataFrame(rows)

# ---------------------------

# Charts

# ---------------------------

def create_line_chart(df: pd.DataFrame, col: str, title_prefix: str = ""):

rule = RULES[col]

plot_df = df[["timestamp", "phase", col]].dropna().copy()

plot_df["severity"] = classify_series(plot_df[col], rule)

plot_df["hover_note"] = [

explain_spike(df, i, col) if plot_df.loc[i, "severity"] != "Normal" else "Normal operating region."

for i in plot_df.index

]

fig = go.Figure()

fig.add_trace(go.Scatter(

x=plot_df["timestamp"],

y=plot_df[col],

mode="lines",

name=rule.display_name,

line=dict(width=2),

))

flagged = plot_df[plot_df["severity"].isin(["Warning", "Critical"])]

if not flagged.empty:

fig.add_trace(go.Scatter(

x=flagged["timestamp"],

y=flagged[col],

mode="markers",

name="Flagged points",

marker=dict(size=9, symbol="diamond"),

text=flagged["hover_note"],

hovertemplate="Time=%{x}<br>Value=%{y}<br>%{text}<extra></extra>",

))

if rule.warn_high is not None:

fig.add_hline(y=rule.warn_high, line_dash="dash", annotation_text="Warn High")

if rule.warn_low is not None:

fig.add_hline(y=rule.warn_low, line_dash="dash", annotation_text="Warn Low")

if rule.crit_high is not None:

fig.add_hline(y=rule.crit_high, line_dash="dot", annotation_text="Critical High")

if rule.crit_low is not None:

fig.add_hline(y=rule.crit_low, line_dash="dot", annotation_text="Critical Low")

title = f"{title_prefix} - {rule.display_name}" if title_prefix else rule.display_name

fig.update_layout(height=430, title=title, xaxis_title="Timestamp", yaxis_title=f"{rule.display_name} ({rule.unit})")

st.plotly_chart(fig, use_container_width=True)

def create_phase_boxplot(df: pd.DataFrame, col: str):

rule = RULES[col]

plot_df = df[["phase", col]].dropna().copy()

if plot_df.empty:

return

fig = px.box(plot_df, x="phase", y=col, color="phase", title=f"{rule.display_name} by Flight Phase")

fig.update_layout(height=380, xaxis_title="Phase", yaxis_title=f"{rule.display_name} ({rule.unit})")

st.plotly_chart(fig, use_container_width=True)

def create_histogram(df: pd.DataFrame, col: str):

rule = RULES[col]

plot_df = df[[col]].dropna().copy()

if plot_df.empty:

return

fig = px.histogram(plot_df, x=col, nbins=40, title=f"Distribution of {rule.display_name}")

fig.update_layout(height=360, xaxis_title=f"{rule.display_name} ({rule.unit})")

st.plotly_chart(fig, use_container_width=True)

def create_correlation_heatmap(df: pd.DataFrame):

numeric_cols = [c for c in RULES if c in df.columns]

if len(numeric_cols) < 2:

st.info("Not enough numeric columns for correlation view.")

return

corr = df[numeric_cols].corr(numeric_only=True)

fig = px.imshow(corr, text_auto=True, aspect="auto", title="Correlation Heatmap of Category II Parameters")

fig.update_layout(height=650)

st.plotly_chart(fig, use_container_width=True)

def create_status_bar(df: pd.DataFrame):

rows = []

for col in RULES:

if col not in df.columns:

continue

sev = classify_series(df[col], RULES[col])

rows.append({

"Parameter": RULES[col].display_name,

"Normal": int((sev == "Normal").sum()),

"Warning": int((sev == "Warning").sum()),

"Critical": int((sev == "Critical").sum()),

})

status_df = pd.DataFrame(rows)

if status_df.empty:

return

melted = status_df.melt(id_vars="Parameter", var_name="Status", value_name="Count")

fig = px.bar(melted, x="Parameter", y="Count", color="Status", title="Parameter-wise Event Severity Count", barmode="stack")

fig.update_layout(height=450, xaxis_tickangle=-45)

st.plotly_chart(fig, use_container_width=True)

def create_anomaly_by_phase(df: pd.DataFrame):

if "anomaly_type" not in df.columns or "phase" not in df.columns:

return

temp = df.groupby(["phase", "anomaly_type"], as_index=False).size().rename(columns={"size": "count"})

fig = px.bar(temp, x="phase", y="count", color="anomaly_type", title="Anomaly Type Distribution by Flight Phase", barmode="group")

fig.update_layout(height=420)

st.plotly_chart(fig, use_container_width=True)

def create_kpi_cards(df: pd.DataFrame):

analytics_score, notes = readiness_score(df)

readiness_score_value, readiness_status = get_readiness_status(df)

crit_total, warn_total = get_grouped_event_totals(df)

c1, c2, c3, c4 = st.columns(4)

with c1:

st.metric("Analytics Score", analytics_score)

with c2:

st.metric("Flight Status", readiness_status)

with c3:

st.metric("Critical Incidents", crit_total)

with c4:

st.metric("Warning Incidents", warn_total)

st.markdown("### Flight Readiness Status")

if readiness_status == "READY FOR FLIGHT":

st.success(f"Flight Status: {readiness_status} | Operational Readiness Score: {readiness_score_value}")

elif readiness_status == "MAINTENANCE REQUIRED":

st.warning(f"Flight Status: {readiness_status} | Operational Readiness Score: {readiness_score_value}")

else:

st.error(f"Flight Status: {readiness_status} | Operational Readiness Score: {readiness_score_value}")

if readiness_status != "READY FOR FLIGHT":

st.markdown("### Why is the aircraft not ready?")

for reason in generate_grouped_readiness_explanation(df):

st.write(f"- {reason}")

root_cause_df = build_root_cause_summary(df)

if not root_cause_df.empty:

st.markdown("### Top grouped root causes")

st.dataframe(root_cause_df.head(5), use_container_width=True)

ranked = rank_root_causes(df)

if not ranked.empty:

st.markdown("### Root Cause Ranking")

top3 = ranked.head(3)

for i, row in top3.iterrows():

label = "Primary Cause" if i == 0 else f"Secondary Cause {i}"

st.write(f"{label}: {row['Parameter']} (Critical: {row['Critical']}, Warning: {row['Warning']})")

st.markdown("### Parameter-Level Explanation")

if notes:

for note in notes:

st.write(f"- {note}")

else:

st.write("- No repeated threshold exceedance pattern was detected in the main parameters.")

# ---------------------------

# UI

# ---------------------------

st.title("✈️ Fighter Aircraft Flight Readiness System")

st.caption("Category II – Fighter / Attack / Trainer")

with st.sidebar:

st.header("Upload Dataset")

uploaded = st.file_uploader("Upload fighter aircraft CSV", type=["csv", "xlsx"])

search_text = st.text_input(

"Search input",

placeholder="Examples: EGT   or   100, EGT   or   6500, fuel",

)

run_search = st.button("Search")

st.markdown("---")

st.write("This app is tuned for the uploaded synthetic telemetry format.")

if uploaded is None:

st.info("Upload your CSV file to start.")

st.code(

"""

Expected sample columns:

- timestamp

- phase

- airspeed_kts

- altitude_ft

- pitch_deg

- roll_deg

- yaw_deg

- engine_rpm_pct

- egt_c

- fuel_flow_kgph

- oil_pressure_psi

- oil_temp_c

- vibration_g

- ambient_temp_c

- anomaly_type

""".strip()

)

st.stop()

if uploaded.name.lower().endswith(".csv"):

df = pd.read_csv(uploaded)

else:

df = pd.read_excel(uploaded)

df = preprocess(df)

create_kpi_cards(df)

st.markdown("### Recommendations")

for item in recommendation_text(df):

st.write(f"- {item}")

report_df = build_report_dataframe(df)

csv_data = report_df.to_csv(index=False).encode("utf-8")

st.download_button(

label="Download Incident Report (CSV)",

data=csv_data,

file_name="flight_readiness_incident_report.csv",

mime="text/csv",

)

if run_search and search_text:

target_value, chosen = parse_value_parameter_search(search_text, df.columns.tolist())

st.markdown("---")

st.markdown("## Search Result")

if target_value is not None and chosen is not None:

hit_rows = find_value_hit_rows(df, chosen, target_value)

create_event_context_charts(df, chosen, target_value, hit_rows)

else:

chosen = search_parameter(search_text, df.columns.tolist())

if chosen is None:

st.warning("No matching column or domain found for the search text.")

else:

create_line_chart(df, chosen, "Search")

flagged = get_flagged_points(df, chosen)

if flagged.empty:

st.success("No threshold spikes detected for the searched parameter.")

else:

show_cols = ["timestamp", "phase", chosen, "severity", "explanation"]

renamed = flagged[show_cols].rename(columns={chosen: f"Value ({RULES[chosen].unit})"})

st.dataframe(renamed, use_container_width=True)

st.markdown("---")

tab1, tab2, tab3, tab4, tab5, tab6, tab7, tab8 = st.tabs([

"Overview",

"Engine Health",

"Flight Attitude",

"Phase Analysis",

"Search Any Parameter",

"Incident Timeline",

"Root Cause Summary",

"Data Preview",

])

with tab1:

create_status_bar(df)

c1, c2 = st.columns(2)

with c1:

if "engine_rpm_pct" in df.columns:

create_histogram(df, "engine_rpm_pct")

with c2:

if "egt_c" in df.columns:

create_histogram(df, "egt_c")

create_correlation_heatmap(df)

create_anomaly_by_phase(df)

with tab2:

engine_choices = [c for c in ["engine_rpm_pct", "egt_c", "fuel_flow_kgph", "oil_pressure_psi", "oil_temp_c", "vibration_g"] if c in df.columns]

selected_engine = st.multiselect(

"Choose engine graphs",

engine_choices,

default=engine_choices[: min(3, len(engine_choices))],

format_func=lambda x: RULES[x].display_name,

key="engine_multiselect",

)

for col in selected_engine:

create_line_chart(df, col, "Engine Health")

flagged = get_flagged_points(df, col)

if not flagged.empty:

st.dataframe(

flagged[["timestamp", "phase", col, "severity", "explanation"]].rename(columns={col: f"Value ({RULES[col].unit})"}),

use_container_width=True,

)

with tab3:

attitude_choices = [c for c in ["pitch_deg", "roll_deg", "yaw_deg", "airspeed_kts", "altitude_ft"] if c in df.columns]

if attitude_choices:

selected_att = st.selectbox(

"Choose flight attitude or profile parameter",

attitude_choices,

format_func=lambda x: RULES[x].display_name,

key="attitude_select",

)

create_line_chart(df, selected_att, "Flight")

c1, c2 = st.columns(2)

with c1:

create_phase_boxplot(df, selected_att)

with c2:

create_histogram(df, selected_att)

flagged = get_flagged_points(df, selected_att)

if not flagged.empty:

st.dataframe(

flagged[["timestamp", "phase", selected_att, "severity", "explanation"]].rename(columns={selected_att: f"Value ({RULES[selected_att].unit})"}),

use_container_width=True,

)

with tab4:

phase_metrics = [c for c in RULES if c in df.columns]

if phase_metrics:

selected_phase_metric = st.selectbox(

"Choose parameter for phase comparison",

phase_metrics,

format_func=lambda x: RULES[x].display_name,

key="phase_metric_select",

)

create_phase_boxplot(df, selected_phase_metric)

create_line_chart(df, selected_phase_metric, "Phase Trend")

with tab5:

typed = st.text_input("Type a parameter or a value search", value="100, egt", key="tab_search")

target_value, chosen = parse_value_parameter_search(typed, df.columns.tolist())

if target_value is not None and chosen is not None:

hit_rows = find_value_hit_rows(df, chosen, target_value)

create_event_context_charts(df, chosen, target_value, hit_rows)

else:

chosen = search_parameter(typed, df.columns.tolist())

if chosen is None:

st.warning("No matching parameter found.")

else:

create_line_chart(df, chosen, "Dynamic Search")

flagged = get_flagged_points(df, chosen)

if flagged.empty:

st.success("No threshold spikes detected.")

else:

st.dataframe(

flagged[["timestamp", "phase", chosen, "severity", "explanation"]].rename(

columns={chosen: f"Value ({RULES[chosen].unit})"}

),

use_container_width=True,

)

with tab6:

st.markdown("### Grouped Incident Timeline")

incident_cols = [c for c in RULES if c in df.columns]

if not incident_cols:

st.info("No valid parameters found for incident analysis.")

else:

selected_incident_col = st.selectbox(

"Choose parameter",

incident_cols,

format_func=lambda x: RULES[x].display_name,

)

grouped = group_consecutive_events(df, selected_incident_col)

if grouped.empty:

st.success("No warning or critical incidents found.")

else:

st.dataframe(grouped, use_container_width=True)

with tab7:

st.markdown("### Root Cause Summary")

root_cause_df = build_root_cause_summary(df)

if root_cause_df.empty:

st.success("No grouped incidents available for root-cause summary.")

else:

st.dataframe(root_cause_df, use_container_width=True)

with tab8:

st.write("Detected columns:", list(df.columns))

st.dataframe(df.head(50), use_container_width=True)